SPECIFICATION:  MALT WHISKY DISTILLING YEAST – “ANCHOR DRY”

Product Description : A naturally pure culture of the yeast, Saccharomyces cerevisiae.

Strain Code : DY 502. Active Dried Distilling Yeast for the Production of Malt Whisky.

Origin : AYCC (Anchor Yeast Culture Collection), S.c. 502 (Natural Strain), Cape Town.

Application : This strain is reccomended for use in the production of malt whisky by fermentation of wort made from malted barley.

Flavour profile : This strain produces a spirit with a highly acceptable flavour profile, as adjudged by Scottish Whisky Manufacturers.

Temperature Range: This strain ferments well over the range of 20 to 33C.

Alcohol Tolerance : This strain displays a good alcohol tolerance of up to at least 15% V/V.

Rehydration : In order to restore the dried yeast to its fully active viable state, it is necessary to rehydrate the yeast as rapidly as possible by adding the yeast to 10 X it's mass of water at 36C with constant vigorous agitation. Stir for 5 minutes before adding the yeast to the washback. ALLOWING THE YEAST TO REMAIN AT 36C FOR LONGER THAN THE STIPULATED TIME PERIOD CAN NEGATIVELY AFFECT ALCOHOL YIELDS.

Analytical :

Total Yeast Count : 1 X 1010 c.f.u. per gram minimum.

Wild Yeast Count: < 105 c.f.u. / g.

Total Bacteria Count: < 106 c.f.u. / g.

Lactobacillus Count: < 106 c.f.u. / g.

Moulds: < 103 c.f.u. / g.

Dry Matter: 92 % minimum.

Packaging : Product packed 2 X 7 kg per 14 kg carton. Product vacuumed packed in laminated (PET / ALUFOIL / PE) liners, in a corrugated cardboard outer carton.

Shelf Life : Stored at 25C and below - 3 years minimum.

Handling :  For optimum shelf life and activity the yeast must be stored under cool dry conditions.